LED Mini Butterfly Light

USER’S MANUAL

Thanks for your choosing our product, please carefully read this manual and operate it obey the specification in order to make sure you can correct and safety installing, operating and maintaining it.

I. Technical Parameters

Voltage: AC90-240V

Frequency: 50/60HZ

Power: 20W

Lamp: 4pcs 3W LEDs

Controller Model: Auto, Sound Activate, DMX512 and Master-slave

Packing Size: 540 *450 *510mm(12pcs)

Net Weight: 1.5KG, Gross Weight: 1.8KG

II. Menu Operation Instructions：

Press “Menu” button to Set，to confirm by press “ENTER” button。correct by pressing “UP&DOWN” button.

| A001 | 001-512 | Function Description |
| --- | --- | --- |
| A1-1 | A1-1～A1-9 | Red color Mode, Rotation speed from slow to fast |
| A2-1 | A2-1～A2-9 | Green color Mode, Rotation speed from slow to fast |
| A3-1 | A3-1～A3-9 | Blue color Mode, Rotation speed from slow to fast |
| A4-1 | A4-1～A4-9 | White color Mode, Rotation speed from slow to fast |
| A5-1 | A5-1～A5-9 | RGB color Gradient mode, Rotation speed from slow to fast |
| A6-1 | A6-1～A6-9 | Gradient mode, Rotation speed from slow to fast |
| A7-1 | A7-1～A7-9 | Gradient mode, Rotation speed from slow to fast |
| A8-1 | A8-1～A8-9 | Strobe mode, Rotation speed from slow to fast |
| S1-1 | S1-1～S1-20 | Sound control model 1 |
| S2-1 | S2-1～S1-20 | Sound control model 2 |

III：DMX Channel Operations:

| Channel | Channel values | Function Description |
| --- | --- | --- |
| CH1 | 0-255 | Dimming |
| CH2 | 0-255 | Red color dimming |
| CH3 | 0-255 | Green color dimming |
| CH4 | 0-255 | Blue color dimming |
| CH5 | 0-255 | White color dimming |
| CH6 | 0-9 | No Function |
|  | 10-255 | Strobe |
| CH7 | 0-5 | No rotation |
|  | 6-127 | Motor positioning go |
|  | 128-255 | Motor rotation speed from slow to fast |
| CH8 | 0-9 | Manual mode: Turn on CH1-CH7 |
|  | 10-72 | RGBW color Fade |
|  | 73-197 | RGBW color Pulse Jump |
|  | 198-220 | Sound Control Model 1 |
|  | 221-255 | Sound Control Model 2 |