7x3W LED PAR LIGHT

USER’S MANUAL

Thanks for your choosing our product, please carefully read this manual and operate it obey the specification in order to make sure you can correct and safety installing, operating and maintaining it.

I. Technical Parameters

Voltage: AC90-240V

Frequency: 50/60HZ

Power: 30W

Lamp: 7pcs*3W  RGB 3 IN 1 LEDs

Controller Model: Auto, Sound Activate, DMX512 and Master-slave

Packing Size: 780 *440 *260mm(10pcs)

Net Weight: 1.1KG, Gross Weight: 1.5KG

II. Menu Operation Instructions：

Press “Menu” button to Set，to confirm by press “ENTER” button。correct by pressing “UP&DOWN” button.

| A001 |  |  | A512 |  |  | 7CH Model |
| --- | --- | --- | --- | --- | --- | --- |
| CP00 |  |  | CP50 |  |  | Fade |
| CC00 |  |  | CC50 |  |  | Auto |
| CJ00 |  |  | CJ50 |  |  | Pulse Jump |
| DEMO |  |  |  |  |  | Demo Model |
| Soud |  |  | Soud |  |  | Sound Control Model |
| RGB |  |  | RGB |  |  | RGB Color |
| ST00 |  |  | ST50 |  |  | Strobe |

| III：DMX Channel Operations: |  |  |
| --- | --- | --- |
| Channel | DMX Value | Function |
| CH1 | 0-255 | Dimming RGBW. From darkness to brightness |
| CH2 | 0-255 | Dimming Red. From darkness to brightness |
| CH3 | 0-255 | Dimming Green. From darkness to brightness |
| CH4 | 0-255 | Dimming Blue. From darkness to brightness |
| CH5 | 0-50 | Strobe and dimming |
|  | 51-99 | G,B Color |
|  | 100-149 | Auto |
|  | 150-199 | Fade |
|  | 200-249 | Pulse Jump |
|  | 250-255 | Sound Control |
| CH6 | 1-255 | Work On CH5 |
| CH7 | 1-255 | Strobe, Speed from slow to fast |