230W Moving Head Beam Light

PLEASE READ THIS CAREFULLY BEFORE USE

Skill parameters

Light source: Osram 7R, 5000 (lm), 8000K

Channel: 16 channels for optical.

Voltage: AC 90V 240V ～(full voltage), 50/60Hz

Power: 350W

Functions:

1) Color wheel: 1 color wheel (14 colors), (8 colors + white) color filters are changeable.

2) Gobo wheel: 1 fixed gobo wheel with 18 changeable gobos.

3) Effect wheel: 1 rotating 8-facet prism, effect movement and gradual fogbow function.

4) 16 millions pixel color LCD, with touch mode and non-touch rolling wheel operating mode, available to display 180° reversely.

5) 0-100% linear mechanical dimmer with mechanical strobe and speed-adjustable strobe effect, support strobe macro function.

6) Electronic linear focusing: beam angle 0°～4°

7) 3-phase electric motor for pan and tilt. Automatic pan and tilt repositioning by photoelectric, repositioning system if accidentally knocked out of place

8) Pan rotation 540° with 8Bit/16Bit resolution, tilt rotation 270° with 8Bit/16Bit resolution

9) Electronic ballast and AC/DC switching power supply. Power factor is up to 0.99, THD<20%. Support lamp power adjustment.

10) Support energy saving mode, lamp-on timing function

11) Superheat protection

12) IP: 20

Dimension: 48.5×43×56.5cm

N.W: 19KG

Channels Control Illustration：

| Item | Channel | DMX address | Function |
| --- | --- | --- | --- |
| CH1 | Pan | 0-255 | 0-450° |
| CH2 | Tilt | 0-255 | 0-270° |
| CH3 | Pan fine | 0-255 | 0--2.1° |
| CH4 | Tilt fine | 0-255 | 0--1.0° |
| CH5 | Speed of pan/tilt | 0-255 | Fast-slow |
| CH6 | Foggy lens | 0-255 |  |
| CH7 | Strobe | 0 | Off |
|  |  | 1-50 | On |
|  |  | 51-240 | Strobe from slow to fast |
|  |  | 241-255 | On |
| CH8 | Dimmer | 0-255 | From dark to bright |
| CH9 | Color | 0-7 | White |
|  |  | 8-15 | Color 1 |
|  |  | 16-23 | Color 2 |
|  |  | 24-31 | Color 3 |
|  |  | 32-39 | Color 4 |
|  |  | 40-47 | Color 5 |
|  |  | 48-55 | Color 6 |
|  |  | 56-63 | Color 7 |
|  |  | 64-71 | Color 8 |
|  |  | 72-79 | Color 9 |
|  |  | 80-87 | Color 10 |
|  |  | 88-95 | Color 11 |
|  |  | 96-103 | Color 12 |
|  |  | 104-111 | Color 13 |
|  |  | 112-127 | Color 14 |
|  |  | 128-191 | Rainbow effect,clockwise from fast to slow |
|  |  | 192-255 | Rainbow effect ,anticlockwise from slow to fast |
| CH10 | Half step of color | 0-255 | Half step of color |
| CH11 | Gobo | 0-6 | Gobo 1 |
|  |  | 7-13 | Gobo 2 |
|  |  | 14-20 | Gobo 3 |
|  |  | 21-27 | Gobo 4 |
|  |  | 28-34 | Gobo 5 |
|  |  | 35-41 | Gobo 6 |
|  |  | 42—48 | Gobo 7 |
|  |  | 49-55 | Gobo 8 |
|  |  | 56-62 | Gobo 9 |
|  |  | 63-69 | Gobo 10 |
|  |  | 70-76 | Gobo 11 |
|  |  | 77-83 | Gobo 12 |
|  |  | 84-90 | Gobo 13 |
|  |  | 91-97 | Gobo 14 |
|  |  | 98-104 | Gobo 15 |
|  |  | 105-111 | Gobo 16 |
|  |  | 112-118 | Gobo 17 |
|  |  | 119-191 | Rainbow effect,clockwise from slow to fast |
|  |  | 192-255 | Rainbow effect,anti clockwise from slow to fast |
| CH12 | Gobo shake | 0 | Off |
|  |  | 1-191 | Fast-slow |
|  |  | 192-255 | Off |
| CH13 | Focus | 0-255 | Near to far |
| CH14 | Prism | 0-31 | Off |
|  |  | 32-255 | Prism |
| CH15 | Prism rotation | 0-127 | Prism manual rotation |
|  |  | 128-191 | Clockwise from fast to slow |
|  |  | 192-255 | Anti clockwise from slow to fast |
| CH16 | Reset | 0-249 | No function |
|  |  | 250-255 | Reset（10s） |

Menu:

There have seven model, Choose menu by pressing ”UP&DWON” button, to confirm by press “ENTER” button

Note: Press “ENTER” button to save each operation

1.DMX Address：

Sets the address code: 001-512

Standard model

1、Control mode

Extension mode

2.Parameters setting

2、Model selection of the light

1、The Pan inversion of control

2、The Tilt inversion of control

3、Pan / Tilt exchange

3.Option             4、The Dimmer reverse channels

5、The Aperture reverse channels

6、The effect of cycle shortest distance

7、Allow DMX reset

1、Display language: Chinese and English

2、Screen brightness: 0-100%

〈1〉Keep bright

4.Display set          3、Backlit Setting    〈2〉Delay darken without operation

〈3〉Delay turn off without operation

4、Show the Logo

5.Manual control: Choose CH1-CH16 adjust the data 0-255

1、Sound control

6.Test run

2、Auto

7.Restore the factory default Settings

8.Rest