120W LED Moving Head Light

User Manual

Please read the enclosed manual before operating this product

Technical Parameters

Electrical: Input Voltage 100~240VAC, 50/60HZ

Light Source: LED Qty 1 x WHITE(120W), Color temperature: 6500k, service life: 50000 hours

DMX：18channels

Pan: 540°/360°  Tilt: 270°  Pan/Tilt: resolution 8-16 bit

Advanced scanning system is more rapid, stable, peaceful, with automatic error correction of the reset function

Color wheel: 7 fixed color wheel +white, Half of color effect, color wheel can be arbitrary orientation, with two-way     rotation of rainbow effect.

Static Gobo: 9 Static gobos +white, Include gobo shake

Rotating Gobo: 7 changeable rotation goboS+white, Include gobo shake

Prism: 3-facet Rotating prism, two-way rotation and prism positioning function

Focus: Electronic focus, Smooth adjust the focal length

Dimmer: 5~100% linear mechanical dimmer

Strobe: 0-13Hz, Can choose random strobe and pulse frequency strobe

Dimmer:0-100% linear mechanical dimmer

Display: LCD display intelligence operations in both English and Chinese

Protection: built-in overheat and high pressure protection

Size: 46.5 x 42 x 35cm

N.W.:13kgs
G.W.:14kgs

DMX Channel Operations:

| Item | Channel | DMX address | Function |
| --- | --- | --- | --- |
| CH1 | Dimmer | 0-255 | rom dark to bright |
| CH2 | Strobe | 0-10 | Off |
|  |  | 11-39 | On |
|  |  | 40-240 | Strobe from slow to fast |
|  |  | 241-255 | On |
| CH3 | Color | 0-15 | Color 1 |
|  |  | 16-31 | Color 2 |
|  |  | 32-47 | Color 3 |
|  |  | 48-63 | Color 4 |
|  |  | 64-79 | Color 5 |
|  |  | 80-95 | Color 6 |
|  |  | 96-111 | Color 7 |
|  |  | 112-127 | Color 8 |
|  |  | 128-191 | Rainbow effect, clockwise from fast to slow |
|  |  | 192-255 | Rainbow effect ,anticlockwise from slow to fast |
| CH4 | Half step of color | 0-255 | Half step of color |
| CH5 | Pan | 0-255 | 0-540° |
| CH6 | Tilt | 0-255 | 0-270° |
| CH7 | Prism | 0-31 | Off |
|  |  | 32-255 | Prism |
| CH8 | Prism Rotation | 0-127 | Prism fixed position |
|  |  | 128-191 | Clockwise from fast to slow |
|  |  | 192-255 | Anti clockwise from slow to fast |
| CH9 | Rotation Gobo | 0-15 | Gobo 1 |
|  |  | 16-31 | Gobo 2 |
|  |  | 32-47 | Gobo 3 |
|  |  | 48-63 | Gobo 4 |
|  |  | 64-79 | Gobo 5 |
|  |  | 80-95 | Gobo 6 |
|  |  | 96-111 | Gobo 7 |
|  |  | 112-127 | Gobo 8 |
|  |  | 128-191 | Rainbow effect, clockwise from slow to fast |
|  |  | 192-255 | Rainbow effect, anti clockwise from slow to fast |
| CH10 | Gobo Rotation | 0-85 | Gobo Rotation t, clockwise from slow to fast |
|  |  | 86-170 | Gobo Rotation, anti clockwise from slow to fast |
|  |  | 171-178 | Gobo static |
|  |  | 179-255 | Gobo shake speed from slow to fast |
| CH11 | Gobo Angle | 0-255 | Gobo Angle adjust |
| CH12 | Static Gobo | 0-12 | Gobo 1 |
|  |  | 13-25 | Gobo 2 |
|  |  | 26-38 | Gobo 3 |
|  |  | 39-51 | Gobo 4 |
|  |  | 52-64 | Gobo 5 |
|  |  | 65-77 | Gobo 6 |
|  |  | 78-90 | Gobo 7 |
|  |  | 91-103 | Gobo 8 |
|  |  | 104-116 | Gobo 9 |
|  |  | 117-127 | Gobo 10 |
|  |  | 128-191 | Rainbow effect, clockwise from slow to fast |
|  |  | 192-255 | Rainbow effect, anti clockwise from slow to fast |
| CH13 | Aperture | 0-199 | Aperture Zoom In/Out |
|  |  | 200-255 | Aperture Zoom In/Out, the speed from slow to fast |
| CH14 | Focus | 0-255 | Near to far |
| CH15 | Pan fine | 0-255 | 16bit scanning |
| CH16 | Tilt fine | 0-255 | 16bit scanning |
| CH17 | Speed of pan/tilt | 0-255 | Fast-slow |
| CH18 | Reset | 0-254 | No function |
|  |  | 255 | Reset（5s） |

Menu:

There have seven model, Choose menu by pressing ”UP&DWON” button, to confirm by press “ENTER” button

Note: Press “ENTER” button to save each operation

1.DMX Address：

Sets the address code: 001-512

Standard model

1、Control mode

Extension mode

2.Parameters setting

2、Model selection of the light

1、The Pan inversion of control

2、The Tilt inversion of control

3、Pan / Tilt exchange

3.Option             4、The Dimmer reverse channels

5、The Aperture reverse channels

6、The effect of cycle shortest distance

7、Allow DMX reset

1、Display language: Chinese and English

2、Screen brightness: 0-100%

〈1〉Keep bright

4.Display set          3、Backlit Setting    〈2〉Delay darken without operation

〈3〉Delay turn off without operation

4、Show the Logo

5.Manual control: Choose CH1-CH16 adjust the data 0-255

1、Sound control

6.Test run

2、Auto

7.Restore the factory default Settings

8.Rest