- LED SPIDER LIGHT

M-YL8-10-4 IN 1

INSTRUCTION MANUAL

Thank you for choosing our LED spider light. For the sake of your safety, Please read and follow these instructions carefully and keep this manual in a safe place for future reference

Menu：

DMX                                                        SLAV

MAST     Menu    down     up     Enter    SOUN

Use button switch and LED display, can set the address code and function code easily.

Check or modify lighting function settings, Press the MENU button. LED display panel will display function menu of lighting. Function operation menu all have corresponding submenu. Each menu all represent the specific functions of lamps, specific see below "operation menu" section.

Technical Parameters:

Voltage: AC90V-240V; 50/60Hz

Power Consumption: 100W (220V)

LED Bulbs: 10W*8LEDs 4in1 LED

LED Current: 1800mA . LED Lifetime: 50000Hours

DMX Channel: 10/36CH

Adjusts the light: 0-100 electron adjusts the light

Strobe: 1-25/second high speed strobe

Head Movement: Tilt 180°,   Beam Angle: 6°

Control mode: DMX 512, Stand –alone mode, Self-test mode, Sound, Master/slave

Control Panel: LED display panel

IP Grade: IP33

Work Environment Temperature: -20°C~40°C

Net weight:7.5KG   Gross Weight: 8.8 kg

Package Size: 73 x 30x 19cm

Menu Function Description：

| ADD | DMX Address | 001 - 512 |
| --- | --- | --- |
| CHND | Channel choose | 10CH / 36CH |
| SLND | NAST/SL1/SL2 | Master-slave mode |
| SHND | Sound effect | SH0 –SH24 |
| SOUN | Sound model | 000 - 100 |
| SENS | Sound sensitivity | Yes - No |
| BLND | Break mode of turn on | Yes - No |
| LED | LED display | On - Off |
| DISP | Display reverse | Yes - No |
| 1TLT | Tilt 1 reverse | Test |
| 2TLT | Tilt 2 reverse | Reset |
| TEST | Test mode | Test |
| FHRS | The time of running | ***hours |
| VER | Program Version | ***** |
| REST | Reset | Yes - No |

Note: When the light turn on, LED display show *****LED-SPIDER-BEAM*****

Keeping The Product Clean:

To ensure the reliability of the projector, it should be kept clean. It is recommended that the fans should be cleaned every 15 days. LED lens should also be regularly cleaned to maintain an optimum light output.

Cleaning frequency depends on the environment in which the fixture operates: damp, smoke or particularly dirty surroundings can cause greater accumulation of dirt on the unit's optics. A soft cloth and typical glass cleaning products should be used for cleaning. It is recommended to clean the external optics at least once every 15 days.

☆Do not use any organic solvent, e.g. alcohol, to clean housing of the apparatus.

Trouble Shooting:

| Problem | Answer |
| --- | --- |
| The LED moving head light doesn’t switch on. | Check the fuse the power socket |
| Although shines, but the LED moving head lamp doesn’t accept the instruction of the controller. | Check the digital start address and also check the connection situation of the communication control line |
| LED moving head lamp only can work intermittently. | Inspect the power supply and voltage is whether normal. |
| Do not receive the DMX512 signal. | Whether the signal lines do contact well or broken. |
| The light beam appears gloomily. | Check that the heat dissipation system is running normally or Inspect the optics part is whether clean. |

☆Other maintain work should by the specialist which has the experience to carry on.

Channel List:

10CH mode:

| Channel | Function | Value |
| --- | --- | --- |
| CH1 | Tilt 1 | 0-255 |
| CH2 | Tilt 2 | 0-255 |
| CH3 | Main dimming | 0-255 |
| CH4 | strobe | 0-255 Speed from slow to fast |
| CH7 | Red dimming | 0-255 |
| CH8 | Green dimming | 0-255 |
| CH9 | Blue dimming | 0-255 |
| CH10 | White dimming | 0-255 |
| CH5 | Macro (Built-in effect program ) | 0-255 |
| CH6 | Macro speed | 0-255 Speed from fast to slow |

36CH mode:

| Channel | Function | Value |
| --- | --- | --- |
| CH1 | Tilt 1 | 0-255 |
| CH2 | Tilt 2 | 0-255 |
| CH3 | Main dimming | 0-255 |
| CH4 | strobe | 0-255 Speed from slow to fast |
| CH5 | R1 dimming | 0-255 |
| CH6 | G1 dimming | 0-255 |
| CH7 | B1 dimming | 0-255 |
| CH8 | W1dimming | 0-255 |
| CH9 | R2 dimming | 0-255 |
| CH10 | G2 dimming | 0-255 |
| CH11 | B2 dimming | 0-255 |
| CH12 | W2 dimming | 0-255 |
| CH13 | R3 dimming | 0-255 |
| CH14 | G3 dimming | 0-255 |
| CH15 | B3 dimming | 0-255 |
| CH16 | W3 dimming | 0-255 |
| CH17 | R4 dimming | 0-255 |
| CH18 | G4 dimming | 0-255 |
| CH19 | B4 dimming | 0-255 |
| CH20 | W4 dimming | 0-255 |
| CH21 | R5 dimming | 0-255 |
| CH22 | G5 dimming | 0-255 |
| CH23 | B5 dimming | 0-255 |
| CH24 | W5 dimming | 0-255 |
| CH25 | R6 dimming | 0-255 |
| CH26 | G6 dimming | 0-255 |
| CH27 | B6 dimming | 0-255 |
| CH28 | W6 dimming | 0-255 |
| CH29 | R7 dimming | 0-255 |
| CH30 | G7 dimming | 0-255 |
| CH31 | B7 dimming | 0-255 |
| CH32 | W7dimming | 0-255 |
| CH33 | R8 dimming | 0-255 |
| CH34 | G8 dimming | 0-255 |
| CH35 | B8 dimming | 0-255 |
| CH36 | W8dimming | 0-255 |